| Parameter spesifikasi | Node Sensor | Cluster Head | LoRa Gateway |
| --- | --- | --- | --- |
| Nama perangkat | Node Sensor | LoRa Cluster Head | LoRa Gateway |
| Fungsi utama | Akuisisi data sensor dan transmisi LoRa | Menerima/mengumpulkan data beberapa node dan meneruskan data | Gateway jaringan LoRa ke server/cloud |
| Model/versi prototipe | Versi 2 | Versi 2 | Versi 2 |
| Mikrokontroler/prosesor | ESP32S3-WROOM-1U | ESP32S3-WROOM-1U | ESP32S3-WROOM-1U |
| Modul LoRa | E22-900MM22S | E22-900MM22S | E22-900MM22S |
| Frekuensi operasi LoRa | 920 MHz | Dual frequensi 920 MHz (star), dan 921 MHz (mesh) | 921 MHz |
| Daya pancar maksimum | 22 dBm | 22 dBm | 22 dBm |
| Bandwidth | 125 KHz | 125 KHz | 125 KHz |
| Spreading Factor | 7 | 7 dan 9 | 9 |
| Jenis antena | omnidirectional | Fiber omnidirectional | Fiber omnidirectional |
| Gain antena | 3 dBi | 3 dBi (star) dan 8 dBi (mesh) | 8 dBi |
| Interface komunikasi lain | SPI dan LoRa | LoRa | LoRa dan Ethernet/Wi-Fi |
| Sensor yang terhubung | MQ-2, MQ-3B, MQ-4, MQ-5, MQ-6, MQ-7B, MQ-8, MQ-135 | – | – |
| Input power | 24 VDC | 5 VDC | 5 VDC |
| Konsumsi daya maksimum | 7.995 W | 0.73 W | 0.73 W |
| Port/konektor eksternal | USB, sensor, power, fan, antena, buzzer alarm | USB, power, antena | Ethernet, USB, power, antena |
| Dimensi | 20 cm × 9 cm × 29 cm | 8 cm × 8 cm × 21 cm | 8 cm × 8 cm × 21 cm |
| Enclosure | metal | metal | metal |
| Mode operasi normal | sensing → processing → transmit | receive → process → transmit | receive LoRa → forward server |

Node sensor

Cluster Head dan/atau Gateway